Laboratory Four

Javier Jesús Macossay-Hernández

EE348L – Electronic Circuits

University of Southern California

Professor Susan Schober

Introduction

In this laboratory, implementation of hand analysis and HSPICE simulations were done to understand the behavior of diodes. Furthermore, the circuits were built in class and different tests were done to them to corroborate our hand calculations and simulation results.

Exercise 1

Procedure

On Exercise 1, a circuit with a resistor and a diode in series was built. A 1 k was used to measure the current, then it was replaced by a 10 k resistor.

Data

Schematic of the circuit

Questions

The current while using the 1 k resistor is 0.46 mA and the diode voltage is 0.536 V. The current while using the 10 k resistor is 0.056 mA and the diode voltage is 0.439 V. The relative change in current decreases by a multiplicative factor (it gets divided by) 8.21 and the diode voltage decreases by a factor of 1.22.

Discussion

The results agree with the estimate in the pre-laboratory exercises because the diode voltage and the current decrease when the resistance value is augmented.

Exercise 2

Procedure

On Exercise 2, the same circuit was built, but the resistor was replaced by a potentiometer. The resistance was varied from 1 k to 10 k, later from 10 k to 100 k, while the current and diode voltage were being monitored.

Data

Table with Values using the Potentiometer from 1 k to 10 k

| 1 k to 10 k | Resistance Value () | VR (Resistor Voltage Drop) | IR (Current) |
| --- | --- | --- | --- |
| 1 | 1.21 | 0.473 | 0.391 |
| 2 | 2.42 | 0.504 | 0.208 |
| 3 | 3.474 | 0.521 | 0.150 |
| 4 | 3.856 | 0.525 | 0.136 |
| 5 | 4.721 | 0.533 | 0.113 |
| 6 | 6.059 | 0.543 | 0.090 |
| 7 | 7.494 | 0.552 | 0.074 |
| 8 | 8.584 | 0.558 | 0.065 |
| 9 | 8.975 | 0.56 | 0.062 |
| 10 | 10.229 | 0.565 | 0.055 |

Table with Values using the Potentiometer from 10 k to 100 k

| 10 k to 100 k | Resistance Value () | VR (Resistor Voltage Drop) | IR (Current) |
| --- | --- | --- | --- |
| 1 | 11.21 | 0.583 | 0.052 |
| 2 | 19.61 | 0.59 | 0.030 |
| 3 | 28.14 | 0.603 | 0.021 |
| 4 | 39.63 | 0.616 | 0.016 |
| 5 | 48.75 | 0.624 | 0.013 |
| 6 | 60.53 | 0.632 | 0.010 |
| 7 | 68.08 | 0.636 | 0.009 |
| 8 | 80.87 | 0.642 | 0.008 |
| 9 | 89.87 | 0.646 | 0.007 |
| 10 | 96.49 | 0.649 | 0.007 |

I-V characteristics using the Potentiometer from 1 k to 10 k

I-V characteristics using the Potentiometer from 10 k to 100 k

Questions

No questions were asked in this exercise.

Discussion

The current and the diode voltage decreased as expected when the resistance value incremented and meeting expectations of the results from Exercise 1.

Exercise 3

Procedure.

On Exercise 3, a circuit with two voltage, AC and DC, sources was built using a 10 k and a diode. Later, the time constant was measured while using different Vbias, DC voltage. The values of Vbias were 0.2 V, 0.4 V, -0.2 V, and -0.4 V. Finally, the capacitance of the diode was calculated using the known value of the resistance for each Vbias.

Data

| Vbias (V) | - 3dB Frequency (Hz) | Vo (mV) | C |
| --- | --- | --- | --- |
| 0.3 | 300 kHz | 212 | 53 pF |
| 0.5 | 230 kHz | 203 | 69 pF |
| -0.3 | 390 kHz | 217 | 44.2 nF |
| -0.5 | 390 kHz | 217 | 44.2 nF |

Questions

According to the results obtained from the laboratory experiment, the capacitance of a diode changes with respect to the bias voltage. The capacitance increase with increasing forward bias and it decreases, but at some point it remains constant, in reverse bias. If the capacitance of the diode is greater than 44.2 nF without applying a voltage bias, then it decreases with reverse bias. The capacitance changes more drastically with forward bias, according to the experimental results.

Discussion

The data, showed in the Data section, matches with the hand calculations, which states that the capacitance increases as Vbias increases, the capacitance is inversely proportional to the reverse bias, and the capacitance changes more drastically under forward bias condition. Therefore, the results agree with the analysis from the pre-laboratory where the same facts were concluded.

Exercise 4

Procedure

On Exercise 4, four different configurations of the circuit were built. In two of them a Vbias, DC, and an AC source were applied and in the other two circuits only an AC source was applied.

Data

Figure 4-14: (a), (b), (c) and (d) for pre-lab exercise 3. R1 = 1 KΩ. The signal source has a frequency of 5 kHz and amplitude of 3V.

Voltage across Resistor for Circuit A

Voltage Input and Voltage Output for Circuit A

Voltage across Resistor for Circuit B

Voltage Input and Voltage Output for Circuit B

Voltage across Resistor for Circuit C

Voltage Input and Voltage Output for Circuit C

Voltage across Resistor for Circuit D

Voltage Input and Voltage Output for Circuit D

Questions

The experimental results are similar to my predictions in the pre-laboratory section. As seen in the Data section, the diode is in forward bias when Vs is positive and reverse bias when Vs is negative. The diode is in reverse bias when Vs is positive and forward bias when Vs is negative. In addition, the diode is in reverse bias when Vs is less than -2 V and it is in forward bias when Vs is greater than -2V. Finally, the diode is in reverse bias when Vs is greater than 2 V and it is in forward bias when Vs is greater than 2 V.

Discussion

The results from the laboratory experiments match with the pre-laboratory results from the hand calculations. Therefore, the hand calculations and predictions were done correctly.

Conclusion

The results clearly agree with the objective of the lab that is to learn how diodes function and how forward and reverse bias conditions affect the behavior of the diode, diode voltage, and current through the circuit. In addition, we use HSpice and WaveView Analyzer to corroborate our hand calculations and our measured values.